# NL5 HTTP Command Usage Examples

All HTTP command examples based on the NL5 documentation are listed below. Note that commands are case-sensitive.

The executive file of NL5 simulator is here:
C:\Users\wynnl\Mains\Personal\AI_projects\Hermes_Projects\Project_NL5_auto\NL5-V4.3\nl5.exe

To turn on NL5 with http port enabled, in CMD  terminal, enter the folder, and use the command:

.\nl5.exe -http

## I. General Functions

- Function: NL5_GetInfo

  - Objective: Retrieve NL5 version information and date.

  - Command: http://localhost/?NL5_GetInfo

- Function: NL5_New

  - Objective: Create a new simulation document and return the document handle.

  - Command: http://localhost/?NL5_New

- Function: NL5_Open

  - Objective: Open the simulation file named "power_stage.nl5".

  - Command: http://localhost/?NL5_Open,power_stage.nl5

- Function: NL5_Save

  - Objective: Save the current active document as "backup.nl5".

  - Command: http://localhost/?NL5_Save,backup.nl5

- Function: NL5_Close

  - Objective: Close the current simulation document.

  - Command: http://localhost/?NL5_Close

- Function: NL5_Show

  - Objective: Activate and display the schematic window for the document with handle 1.

  - Command: http://localhost/?NL5_Show,1

- Function: NL5_GetActive

  - Objective: Get the handle of the current active document.

  - Command: http://localhost/?NL5_GetActive

- Function: NL5_GetParam

  - Objective: Get the resistance value of component R1.

  - Command: http://localhost/?NL5_GetParam,R1.R

- Function: NL5_SetParam

  - Objective: Set the capacitance of component C1 to 10uF.

  - Command: http://localhost/?NL5_SetParam,C1.C=10u

- Function: NL5_DisableCmp

  - Objective: Disable components S1 and S2.

  - Command: http://localhost/?NL5_DisableCmp,S1,S2

- Function: NL5_EnableCmp

  - Objective: Enable component D1.

  - Command: http://localhost/?NL5_EnableCmp,D1

- Function: NL5_SetCmpState

  - Objective: Set the initial state of switch Q1 to "short".

  - Command: http://localhost/?NL5_SetCmpState,Q1,short

## II. Transient Simulation Instructions

- Function: NL5_GetTracesSize

  - Objective: Get the total number of traces in the current transient simulation.

  - Command: http://localhost/?NL5_GetTracesSize

- Function: NL5_AddTrace

  - Objective: Add a current signal trace for R1.

  - Command: http://localhost/?NL5_AddTrace,I,R1

- Function: NL5_DeleteTrace

  - Objective: Delete the transient trace at index 2.

  - Command: http://localhost/?NL5_DeleteTrace,2

- Function: NL5_GetTrace

  - Objective: Get the index and handle of the trace named "V(out)".

  - Command: http://localhost/?NL5_GetTrace,V(out)

- Function: NL5_GetTraceValue

  - Objective: Get the peak-to-peak (pp) value of trace "V(L1)".

  - Command: http://localhost/?NL5_GetTraceValue,V(L1),pp

- Function: NL5_SetTrace

  - Objective: Set the color of the trace at index 0 to red.

  - Command: http://localhost/?NL5_SetTrace,0,color=red

- Function: NL5_ShowTran

  - Objective: Open the transient waveform display window.

  - Command: http://localhost/?NL5_ShowTran

- Function: NL5_SetScreen

  - Objective 1: Set the waveform window horizontal start time to 0.1s and display span to 0.01s.

  - Command 1: http://localhost/?NL5_SetScreen,start=0.1,screen=0.01

  - Objective 2: Separate all waveforms vertically, using individual vertical axes.

  - Command 2: http://localhost/?NL5_SetScreen,separate=on

- Function: NL5_Cursors

  - Objective: Enable cursors and place the left cursor at the 0.05s position.

  - Command: http://localhost/?NL5_Cursors,on,left=0.05

- Function: NL5_Start

  - Objective: Start transient simulation with a step size of 1u.

  - Command: http://localhost/?NL5_Start,step=1u

- Function: NL5_Pause

  - Objective: Pause the ongoing transient simulation.

  - Command: http://localhost/?NL5_Pause

- Function: NL5_Continue

  - Objective: Continue simulation and extend the simulation window duration to 0.2s.

  - Command: http://localhost/?NL5_Continue,screen=0.2

- Function: NL5_Stop

  - Objective: Stop the current transient simulation.

  - Command: http://localhost/?NL5_Stop

- Function: NL5_SaveIC

  - Objective: Save the initial conditions of the current simulation.

  - Command: http://localhost/?NL5_SaveIC

- Function: NL5_SaveAsWithIC

  - Objective: Save as a new file "state.nl5" and save the current state as the initial condition for that file.

  - Command: http://localhost/?NL5_SaveAsWithIC,state.nl5

- Function: NL5_IsRunning

  - Objective: Check simulation status and current simulation time.

  - Command: http://localhost/?NL5_IsRunning

- Function: NL5_GetDataSize

  - Objective: Get the number of transient data points generated for trace "V(C1)".

  - Command: http://localhost/?NL5_GetDataSize,V(C1)

- Function: NL5_GetDataAt

  - Objective: Retrieve 50 raw data points of trace 0 starting from the 100th point.

  - Command: http://localhost/?NL5_GetDataAt,0,start=100,points=50

- Function: NL5_GetData

  - Objective: Get interpolated data for trace "V(out)" between 0s and 0.01s with a step of 10u.

  - Command: http://localhost/?NL5_GetData,V(out),from=0,to=0.01,step=10u

- Function: NL5_AddData

  - Objective: Add a time/value pair (t=1, v=2.5) to a data-type trace.

  - Command: http://localhost/?NL5_AddData,Data1,1,2.5

- Function: NL5_Export

  - Objective: Export data for specific traces (V(R1), I(R2)) to "result.csv" from 1s to 3.3s with a 0.05s step.

  - Command: http://localhost/?NL5_Export,result.csv,V(R1),I(R2),from=1,to=3.3,step=0.05

- Function: NL5_GetStorageSize

  - Objective: Get the number of stored historical data sets for transient simulation.

  - Command: http://localhost/?NL5_GetStorageSize

- Function: NL5_Store

  - Objective: Store the current simulation data in a storage area named "Case1".

  - Command: http://localhost/?NL5_Store,Case1

- Function: NL5_DeleteStorage

  - Objective: Delete all saved historical data.

  - Command: http://localhost/?NL5_DeleteStorage

## III. AC Simulation Instructions

- Function: NL5_GetACTracesSize

  - Objective: Get the total number of AC traces.

  - Command: http://localhost/?NL5_GetACTracesSize

- Function: NL5_AddACTrace

  - Objective: Add an impedance (Z) signal trace for component C1.

  - Command: http://localhost/?NL5_AddACTrace,Z,C1

- Function: NL5_DeleteACTrace

  - Objective: Delete the AC trace named "Z(L1)".

  - Command: http://localhost/?NL5_DeleteACTrace,Z(L1)

- Function: NL5_GetACTrace

  - Objective: Get AC trace information based on index 1.

  - Command: http://localhost/?NL5_GetACTrace,1

- Function: NL5_GetACTraceValue

  - Objective: Get the value of AC trace 0 at 1000Hz.

  - Command: http://localhost/?NL5_GetACTraceValue,0,1000

- Function: NL5_SetACTrace

  - Objective: Set AC trace properties, such as hiding all AC traces.

  - Command: http://localhost/?NL5_SetACTrace,all,show=off

- Function: NL5_ShowAC

  - Objective: Display the AC simulation results window.

  - Command: http://localhost/?NL5_ShowAC

- Function: NL5_SetACScreen

  - Objective: Set the AC window to display dB on the vertical axis and set the start frequency to 10Hz.

  - Command: http://localhost/?NL5_SetACScreen,left=10,db=on

- Function: NL5_ACCursors

  - Objective: Display cursors in the AC window and set the right cursor position to 100k.

  - Command: http://localhost/?NL5_ACCursors,on,right=100k

- Function: NL5_StartAC

  - Objective: Specify V1 as the AC source and run a 100-point AC simulation from 10Hz to 1MHz.

  - Command: http://localhost/?NL5_StartAC,source=V1,from=10,to=1M,points=100

- Function: NL5_StopAC

  - Objective: Stop the current AC simulation.

  - Command: http://localhost/?NL5_StopAC

- Function: NL5_GetACDataSize

  - Objective: Get the number of data points for AC trace at index 0.

  - Command: http://localhost/?NL5_GetACDataSize,0

- Function: NL5_GetACDataAt

  - Objective: Retrieve the frequency, magnitude, and phase of the first data point for trace "Z(C1)".

  - Command: http://localhost/?NL5_GetACDataAt,Z(C1),start=0,points=1

- Function: NL5_GetACStorageSize

  - Objective: Get the number of stored historical AC data sets.

  - Command: http://localhost/?NL5_GetACStorageSize

- Function: NL5_StoreAC

  - Objective: Store the current AC simulation results in the storage area.

  - Command: http://localhost/?NL5_StoreAC

- Function: NL5_DeleteACStorage

  - Objective: Delete the specified 1st group of stored historical AC data.

  - Command: http://localhost/?NL5_DeleteACStorage,1